WizMark v3.0 Results

16 April, 1997

Copyright ã 1996, 1997 3Dfx Interactive, Inc. All Rights Reserved

3Dfx Interactive, Inc.

4435 Fortran Drive

San Jose, CA  95134

Phone:  (408) 935-4400

Fax:  (408) 262-8874

- About WizMark  (the story)

Can  your 3D board deliver high quality rendering without sacrificing performance??

WizMark is a 3D performance benchmark created by 3Dfx Interactive, Inc. that measures the performance of Microsoft's Direct3D™ accelerated by 3D hardware.  The benchmark utilizes essential 3D rendering features which provide enhanced visual realism.  Some of these features include:  Gouraud shading, perspective correct texture-mapping, Gouraud modulated texture mapping, texture bilinear filtering, texture LOD MIP-mapping, Alpha-blending, and Z-buffering.

WizMark is based on models and artwork from 3Dfx Interactive’s "Wizard's Tower," a first person perspective walk through of a series of connected rooms. WizMark is more accurate at measuring sustainable 3D performance than other synthetic 3D benchmarks (like the infamous spinning teapot) because it is derived from an actual real-world application.  It is also designed to stress both triangle and fill rate limits of the graphics accelerator. WizMark records the time it takes a given Direct3D hardware accelerator to render each frame as the camera navigates a predefined path through a typical Wizard's Tower room.  The resulting WizMark score is a scaled average of the framerate measured while rendering each of the 3D scenes.  WizMark provides warning messages if the 3D board being tested does not support some of the 3D features that WizMark uses.  The level of image quality rendered by the 3D board dramatically decreases if such features are missing.

WizMark v3.0

Currently, only the Verite', Rage II and Voodoo Graphics chipsets are able to run WizMark in "full" mode.  All of the other 3D graphics accelerators have to run WizMark in "lite" mode because they fail to support one or more of the key 3D features required for fully immersive and interactive 3D gaming environments.  Using these advanced features will have an effect on the performance of most product offerings.  The Voodoo Graphics chipset from 3Dfx Interactive offers full 3D features at the highest performance, NO COMPROMISES!!!

WizMark v3.0 is an updated version that allows you to capture data from multiple benchmark tests from a single command line prompt.  By allowing you to output the results of multiple runs to a specified file you can collect true statistical benchmark data about your 3D graphics accelerator.

II.	How to use WizMark  (we made it easy)

WizMark should be executed via a DOS command line.  Since there are only very few 3D graphics accelerators available in the market today that can support advanced features, we found the necessity to add command line options for WizMark.  These options allow users to have an "apples to apples" performance comparison between full featured 3D products like Voodoo Graphics from 3Dfx Interactive and those products that can only support a minimal subset.

The following WizMark options are available:

1)	WIZMARK (no options)

Executes only if the graphics accelerator supports all of the advanced features.

2)	WIZMARK -lite

Executes the benchmark without lighting (meaning no lighting maps, no Gouraud modulated texturing and no Alpha Blending).  Allows users to compare full-featured boards with those boards that have only a subset of the 3D features required for realistic and high impact visual effects.

3)	WIZMARK -software

Executes the test of the software subsystem and runs in "lite" mode.  Hardware acceleration is not used.

4)	WIZMARK -front

Executes the benchmark test but renders to the front buffer only.  This basically eliminates 
V-Sync and gives you the true raw performance of the 3D graphics accelerator.  Obviously the visual effects will not be very good, but this test provides you with a very good test of baseline performance.  Can be used in “full” mode if the card supports it as well as “lite” mode.

5)	WIZMARK -output <filename>

This will cause WizMark to run and dump the statistical data to a file specified by <filename>, (remember to omit the “<>”’s).  Data collected will be the number of frames, elapsed time, frames per second, and the WizMark benchmark value.

6)	WIZMARK -times <n>

This will cause WizMark to run the number of times indicated by <n>, (remember to omit the “<>”’s).  WizMark will capture all the data as specified by the -output option as well as an average benchmark value for all the runs.  If you use this command line option and do not specify an output file, WizMarks will default to STATS.TXT.

III.	Data Collection Procedures  (our lawyers insisted)

Note that it is important not to compare the WizMark "full" mode result against the WizMark "lite" results.  The WizMark "full" mode has been setup to use models and scenes that contain significantly more triangles than the "lite" version.  TO THAT END, THE RESULTS OBTAINED FROM WIZMARK "FULL" AND WIZMARK "LITE" CANNOT BE COMPARED TO ONE ANOTHER AS THE RESULTS ARE CALCULATED USING DIFFERENT SCALE FACTORS.

Minimum system, software and 3D graphics board component requirements

16 MB system memory, 1.5 MB texture memory, Z-buffer and 2MB memory for display, Windows® 95 and DirectX™ 3.0A and Direct3D™

Boards / drivers  used to obtain the enclosed results:

| Vendor / Product(chipset) | Direct3D Driver Version |
| --- | --- |
| ATI 3D Expression+ PC2TV™ (3D Rage® II) | 4.03.2501 |
| Canopus Total3D™ (Verite’™) | 4.03.00.9001-1.13 |
| Creative Labs 3D Blaster™ (Verite’) | 4.02.0206 |
| Intergraph® Reactor 3D (Verite’) | 4.03.11.13 |
| Matrox Mystique® (Matrox) | 4.03.00.3410 |
| VideoLogic Apocolypse3D (PowerVR™) | 1.03C1 |
| Sierra Screamin’ 3D™ (Verite’) | 4.03.00.9001-1.12 |
| Symmetric GLyder Accelerator (Permedia™ + Delta™) | 4.03.00.2101-0076 |
| Alliance Promotion AT3D™ (Voodoo Rush™) | 4.03.2037 |
| Trident ProVidia 9685-1™ (Voodoo Rush) | 4.04.666 |
| 3Dfx Interactive reference board (Voodoo Graphics™) | 4.03.00.2108 |

Test Machines

P5-166 clone, 32 MB RAM, 256K SRAM

HP Vectra XA6 / 200DT:  Pentium Pro 200, 64MB RAM, 256K SRAM

All data was collected using the average of three runs using the “-times 3” command line option.  The standard deviation of all runs were well within statistical norms.

A best effort was put forth to obtain the latest drivers and BIOS updates for all competitor products.

IV.	Results  (because you deserve them)

Chart #1

The following results were achieved by using WizMark v3.0.  WizMark was executed in full mode without any options using a Pentium system running at 166 MHz.

Chart #2

The following results were achieved by using WizMark v3.0.  WizMark was executed in full mode without any options using a Pentium Pro system running at 200 MHz.

Chart #3

The following results are derived by using WizMark v3.0  in “lite” mode (WizMark -lite) using a Pentium system running at 166 MHz.

Note:  The Apocalypse 3D and Mystique products  do not support Bi-linear filtering which affects the quality of the images displayed on the screen.  We recommend that you run WizMark first-hand in order to examine the difference in the quality of the image without some of these key 3D features “turned on”.

Chart #4

The following results are derived by using WizMark v3.0 in “lite” mode (WizMark -lite) using a Pentium Pro system running at 200 MHz.

Note:  The Apocalypse 3D and Mystique products  do not support Bi-linear filtering which affects the quality of the images displayed on the screen.  We recommend that you run WizMark first-hand in order to examine the difference in the quality of the image without some of these key 3D features “turned on”.

The Promotion AT3D is a high-performance integrated 3D, 2D, and video accelerator featuring the Voodoo Rush 3D chipset.  AT3D is being developed in a partnership between Alliance Semiconductor and 3Dfx Interactive and as of print of this document it was in final production.

The ProVidia 9685-1 is a high performance integrated 3D, 2D, and video accelerator featuring the Voodoo Rush 3D chipset.  ProVidia is being developed in a partnership between Trident Microsystems and 3Dfx Interactive and as of print of this document it was in Alpha.

Voodoo Graphics and Voodoo Rush are trademarks of 3Dfx Interactive, Inc.  All other trademarks are the property of their respective owners.